Tutorial Sheet-6

1. Which data transfer scheme is most suitable for high-speed devices like disks?

A) Polling
B) Programmed I/O
C) DMA
D) Manual transfer

Answer: C) DMA

2. The purpose of an interrupt mask is to:

A) Increase memory speed
B) Disable selected interrupts
C) Clear cache memory
D) Enable DMA transfers

Answer: B) Disable selected interrupts

3. Which UART register stores received serial data after conversion?

A) Shift register
B) Status register
C) Receiver buffer register
D) Control register

Answer: C) Receiver buffer register

4. During DMA transfer, CPU:

A) Stops permanently
B) Cannot execute any instruction
C) May continue executing instructions
D) Clears RAM contents

Answer: C) May continue executing instructions